INVADE BIO DRAIN

KEY INFORMATION

Pesticide Type:		Not Applicable

Pesticide Group: 		Not Applicable

Active Constituent:		Bacillus spp., citrus oil

Concentration:		594 billion cfu per .95 litres

FORMULATION

The product is composed of a premium blend of waste digesting bacillus spores, and citrus oil in a thickened formula.

PEST CONTROLLED

Larval stages of fruit, drain and phorid flies through breeding site elimination.

SITUATION

Domestic, Commercial, Industrial and Food Handling Establishments.

MICROBIAL BLEND

The microbes used in InVade Bio products are a multiple spore blend designed to provide exceptional performance over a wide range of organic waste-related applications, including drain line and grease trap treatment, hard surface cleaning and deodorizing, septic and waste treatment, and carpet and fabric care. The strains were selected for their superior enzymatic activity against specific substrates including proteins, carbohydrates, and fats and their combined synergistic value as demonstrated in enhanced cell growth, germination rate and effectiveness over a wide range of organic substrates. They will perform in both aerobic and anaerobic environments. Spore-formed microbes are superior in that they are more resistant to chemical cleaning agents. When the spores are in contact with their food source (the target organic matter), they generate bacterial colonies. These bacteria then digest organic matter. While the bacteria can be killed by chemicals, the spores survive and can again produce bacterial colonies when conditions become favourable ( e.g. the chemical is rinsed away).

The spores consist of Bacillus strains (multiple), with no detectable Salmonella or Shigella. Strains are included which produce Lipase, Protease, Amylase, Cellulase and Urease enzymes with aerobic and facultative anaerobic pathways. They are effective at a temperature range of 38 – 145 degrees F. In laboratory tests, the microbe counts double in 20 minutes when in contact with their food sources. They will perform in a pH range of 4.5 to 8.5. They are stable for two years if stored in the range 35 – 95 degrees F.

APPLICATION

Moisten drain with warm water ( 5 – 10 litres) before treatment if not used regularly and apply at low drain usage times of the day. Apply about 100mL to each drain around the edges to coat the sides. Repeat daily for 5 – 7 days and then every 1 – 2 weeks for ongoing maintenance. For drains that have a very heavy layer of scum, clean the drain with a brush before beginning treatment for immediate results and then maintain with InVade Bio Drain to prevent further build-up.

MODE OF ACTION

The premium microbes eat through the scum and the citrus oil provides natural degreasing power.

SPECIAL COMMENTS

The premium microbes also quickly knock out the noxious odour-causing bacteria, resulting in excellent odour control fast.